    - Packing & Packing Materials

Before packing any goods, it is essential to be aware of what the risks are, and what protective measures you need to take for any item to protect it from the risks to which it is particularly vulnerable.  Risks fall into six categories:

- Shock:  caused when goods or container are dropped, or hit other items, or collide with other moving vehicles.  Particularly vulnerable to shock are breakable goods, china, glass, marble and delicate furniture.  Protective measures including padding material, cartons, interior cases and careful handling at all the times.

- Vibration or Pressure:  this arises during all forms of transport, particularly on rough roads.  The combination of weight and vibration can cause a sharp edge or pint to ear through even the thickest protective warping.  Particularly vulnerable are polished surfaces, inadequately packed cartons which can be compressed, and loose or baldy loaded goods which can change position during transit.  Protective measures include plenty of wrapping materials, tight packing of cartons and secure loading of vehicles and containers, fill the gaps, secure goods at all stages, heavy items below, light items above, flat surfaces next to other flat surfaces.

- Weather:  Rain which falls on unprotected cases or finds its way into a vehicle or container causes damage.  Extremes of heat and cold can cause condensation and mildew.  Use absorbent paper wrapping next to polished surfaces, not polythene or grease-proof paper.

- Security:  Some goods are more likely to be mislaid or stolen than others.  Money, jewellery and stamp collections are examples, and the best precaution is to advise the client to make his own arrangements for them. Or to make a complete detailed inventory, check carefully and note well the conditions.

- Dirt:  dirty hands make marks on clothing, bedding and upholstered furniture.  Dirty feet walking in and out of the house of a wet day can mark carpets and floor surfaces.  Dust on road and in warehouses can permeate through the smallest cracks and soil furniture.  Precautions include to wash your hands before handling these goods, put down floor runners to protect the floors and make sure container and vehicle doors are properly sealed.  Wear gloves to handle these items is another solution.

- Dismantling:  some items have to be dismantled before removal, either so that they can be properly protected, or because they are too big or bulky to be moved whole.  Seldom are they re-assembled by the same person who dismantled them, so certain precautions have to be taken.  Individual parts should be packed together in the same case, or clearly marked, and screws, bolts, keys etc. Should be put in a bag and attached or taped to the main piece of furniture.

The time for greatest risk to the goods are:  packing, carrying, loading, unloading, unpacking.

The success and effectiveness of a good packing job at the client’s home depends very much on the way the packers organize his work and the goods to be packed.  There are a few rules to be considered in any packing job:

- Carpets must be rolled and put aside

- Put all the furniture together

- Make an early start on packing the kitchen as this tends to be very time consuming.

- Be careful with any type of food or liquor that must not be included in the shipment.

- Mark the cartons as you pack them.  When marking, make sure you understand the reasons for marking and listing:  enable to clear customs, enable to establish a receipt as part of the B/L and for the client, enable to identify the different items after packing and the condition before the transport.

- Keep a supply of plastic bags for all the garbage

- Keep unused packing material in a neat particular location

- Before you can wrap some furniture, you may have to lay it down on the floor.  Always put a blanket on the floor to provide some protection.

- Remember to use empty space in the furniture

- Always have soft paper closest to the furniture, never use corrugated paper.

- Make sure your hands are clean when handling dresses, suits

- Ask the client if you are uncertain

- Use some common sense when packing household goods.

What are the most commonly packing materials needed?

- Cartons of different sizes include: small, medium, large, china carton with dividers, wardrobes with hangers, mirror or picture cartons

- Soft wrapping paper for china and delicate items

- Kraft paper for wrapping and protect surfaces

- Foam or equally soft material for wrapping

- Corrugated cardboard or paper pads for outside wrapping of furniture

- Tape and a special holder for the tape roll

- Labels, markers and packing lists

    - Loading and Preparation for shipping

When loading the goods, first thing is to observe weight limitations.  So there are some rules to apply:

- Don’t exceed the rated capacity of the container or lift van.  Distribute the weight equally.

- Avoid concentrating heavy weights at one side or at one end.  Stow the heaviest items on the bottom.  Heavy, dense items should be crated to distribute weight.

- Be sure to observe hazardous material regulations

- Have all goods ready before loading begins

- Use waterproof linings inside the container to protect goods on their tops and sides to minimising loss from water damage

- Plan for ease of unloading

- By their very nature, household goods will not always fill a container completely and so there will be a tendency for them to shift.  To avoid this, you must secure the shipment by cushioning, blocking, bracing or anchoring the goods.

- Make sure that all the locking lugs are engaged and attached all locks and seals.

    - Storage

Permanent storage means storage of the goods for a longer, often unspecified period of time.  The charges are calculated either per floor space, per volume, per weight or per pallet or storage container mostly on a monthly basis.

When there is s storage element in an international move, it is always treated as being quite separate, with the storage company having a distinct liability for the extent and duration of the warehousing.

This is expressed in a storage contract that is established at the time of taking the goods into the warehouse.  Make absolutely certain that you establish a storage contract which is entirely separate from the international moving contract.

Storage in transit (SIT) is by contrast, part of the international move and occurs either before or after shipment.  It is a service which is described and made part of the removal contract.  The liability for care of the goods remains the booker’s responsibility as specified in the contract.  This is the reason why the same liability conditions that govern the total move include the period of temporary storage.  SIT rates are shown in the rate schedule of most international moving companies.

When the shipment is taken into SIT, the destination agent must inform the booker immediately as the will be responsible for extending the insurance covers.

Storage under customs bond (bonded warehouse) is where customs authorities allow them to store goods not yet cleared through customs.  A bonded warehouse must need specific requirements, especially regarding security.  Storage charges for this facility tend to be higher than for conventional storage.